HPLC method development for analysis of Flutamide: Application to protein binding studies

Abstract

The main goal of this study was development of an analysis method for flutamide quantitation which is applicable to protein binding studies. Ultrafilteration method was used for protein binding study of flutamide. For sample analysis, flutamide was extracted by a simple, low cost extraction method using diethyl ether and then determined by HPLC/UV detection. Acetanilide was used as an internal standard. The chromatographic system consisted of a reversed-phase C8 column with C8 pre-column, an isocratic mobile phase of potassium dihydrogen phosphate buffer with pH adjusted to 3.2-methanol-acetonitril (33/29/38 v/v) and UV detection at 226.4 nm. Acetanilide and flutamide were eluted at 1.8 and 2.9 min, respectively. The method was linear over the concentration range of 62.5-16000 (r2 > 0.99). The limit of quantification of method was 62.5 ng/ml. The intra-day and inter-day (precision and accuracy) ranges were found to be (0.2-1.4 %, 90-105 %) and (0.2-5.3 %, 86.71-98.49 %) respectively. Acetanilide and flutamide capacity factor values of 1.35 and 2.87, tailing factor values of 1.24 and 1.07 and resolution values of 1.8 and 3.22 were obtained.Based on the obtained results a rapid, precise, accurate, sensitive and cost-effective RP-HPLC method was proposed for quantitative determination of flutamide.