Development and validation of HPLC/UV method for determination of flutamide: application to protein binding studies

Sara Esmaeelzadeh, Hadi Valizadeh, Parvin Zakeri-Milani

Purpose: The main goal of this study was development of an analysis method for flutamide quantitation which is applicable to protein binding studies. Methods:  Ultrafilteration method was used for protein binding study of flutamide. For sample analysis, flutamide was extracted by a simple, low cost extraction method using diethyl ether and then determined by HPLC/UV detection. Acetanilide was used as an internal standard. The chromatographic system consisted of a reversed-phase C8 column with C8 pre-column, an isocratic mobile phase of potassium dihydrogen phosphate buffer with pH adjusted to 3.2-methanol-acetonitril (33/29/38 v/v) and UV detection at 226.4 nm. Results: Acetanilide and flutamide were eluted at 1.8 and 2.9 min, respectively. The method was linear over the concentration range of 62.5-16000 (r2 > 0.99). The limit of quantification of method was 62.5 ng/ml. The intra-day and inter-day (precision and accuracy) ranges were found to be (0.2-1.4 %, 90-105 %) and (0.2-5.3 %, 86.71-98.49 %) respectively. Acetanilide and flutamide capacity factor values of 1.35 and 2.87, tailing factor values of 1.24 and 1.07 and resolution values of 1.8 and 3.22 were obtained. Conclusion: Based on the obtained results a rapid, precise, accurate, sensitive and cost-effective RP-HPLC method was proposed for quantitative determination of flutamide.

Keywords: Flutamide, Acetanilide, Protein binding, extraction, HPLC, UV.